UFAM Doppler Lidar NetCDF Documentation

Written by Karen Bozier, UFAM Instrument Scientist

16th November 2004

1.  Introduction

The data set consists of parameters measured by the UFAM (UK Universities Facilities for Atmospheric Measurements) Doppler lidar system, which is operated and maintained by the University of Salford.  The data set was collected during the Improved Air Quality Forecasting project (ISB52) field campaign which took place during July 2003.  The UFAM Doppler lidar was situated at RAF Northolt, West London, UK (51:32:59 ºN, 0:24:26 ºW).  For more information on the ISB52 project please visit the projects web page: http://badc.nerc.ac.uk/data/isb52/.  The general characteristics of the Doppler lidar system are given below.

Parameters				Description/Value

Operating wavelength			10.6 m

System Intermediate Frequency	12 MHz

Energy per pulse			50 mJ

Pulse length				0.6 sec

Pulse Repetition Frequency		up to 50 Hz

Nominal range gate			112 m

Maximum range	up to 9 km

Minimum range			500 - 700 m

For more information regarding the UFAM Doppler lidar system please visit the following web addresses:

http://ufam.nerc.ac.uk/ufam/ - for information on UFAM ,

http://www.ties.salford.ac.uk/people/keb/ - for information on the Doppler lidar system and current research within the Hydrometeorology and Remote Sensing Group at the University of Salford.

If you have any problems obtaining the data, then please contact the British Atmospheric Data Centre (BADC – http://badc.nerc.ac.uk/home/index.html).  If you have any problems, queries or comments regarding the data themselves, that are not covered adequately by this document, then please contact Dr Karen Bozier, email: k.bozier@salford.ac.uk.

2.  NetCDF Data format

The data are in NetCDF format (information can be found regarding this file format on the BADC website (http://badc.nerc.ac.uk/help/formats/netcdf/).

The following filename convention is used for each data file:

Instrument_location_YYYYMMDDhhmmss_sss.nc

where instrument is salford-lidar-dop-1, location is northolt and YYYMMDDhhmmss indicates the date and time of the start of the scan.  sss is the scan type, which will be either ppi (plan position indicator, a horizontal scan), rhi (range height indicator, a vertical scan) or fix (a constant azimuth and elevation scan).

An example file name is:  salford-lidar-dop-1_salford_20031218125913_fix.nc where the data was taken in Salford at 12:59:13 UTC on 18th December 2003 with the elevation and azimuth angles fixed.

All NetCDF files contain the following variables:

latitude – the latitude of the Doppler lidar (degrees north),

longitude – the longitude of the Doppler lidar in (degrees east),

altitude – the altitude of the lidar system above mean sea level (m),

wavelength – the wavelength of the Doppler lidar system (10.6 m),

pulse_delay – the time delay between lidar pulses (x 10-3 s),

gate – the number of range gates for each set of line of sight (LOS) measurements.  Depending on the setting of the maximum range, the gate number will be set to 20, 40 or 80 which corresponds to a maximum of range of 2184 m, 4424 m or 8904 m,

range_resolution – the distance between each range gate (112 m),

integrations – the number of pulses integrated for each set of line of sight (LOS) measurements,

time – a vector containing the time for the start of each line of sight (LOS) measurement in decimal hours UTC.  The time is recorded in decimal hours from midnight UTC to the middle of each ray,

range – a vector containing the range from the lidar to the centre of each range gate,

elevation – a vector containing the centred elevation angle of each line of sight (LOS) measurement above the horizon (degrees),

azimuth – a vector containing the centred azimuth angle of each line of sight (LOS) from due north (degrees),

intensity – a vector containing the amount of backscattered signal received at the detector.  This parameter is an arbitrary measurement,

radial_velocity – a vector containing the estimated Doppler velocity (m s-1) measured along the line of sight (LOS) of the lidar beam.  Positive velocities indicate airflow away from the lidar; negative velocities indicate airflow towards the lidar.  Note that no noise has been removed from this parameter.

corrected_radial_velocity – a vector containing the estimated Doppler velocity       (m s-1) measured along the line of sight (LOS) of the lidar beam.  Positive velocities indicate airflow towards the lidar, negative velocities indicate airflow away from the lidar.  Note that noise has been removed from this parameter.  Noise has been removed from the data and a value of -999.00 (indicated by the missing_value and _FillValue attributes) in the data indicates that the velocity estimates were deemed to be noise.  A value of -999.00 was assigned to velocity estimates that fell within the minimum range of the system measurements or within the system noise. The limit for good velocity estimates was calculated using the intensity parameter and calculating the SNR (signal to noise ratio).  Velocity estimates that do not have an SNR value greater than -14 dB are designated as being within the noise level of the system.  In practise, this noise removal algorithm can remove velocity estimates that appear to be ‘good’ but have an SNR of less than -14 dB.

The following global attributes are present:

Conventions – a string indicating that the data conform to the Climate and Forecast (CF) metadata conventions (“CF-1.0”),

title – a string describing what the data set is,

institution – a string specifying where the original data was produced,

source – a string specifying the method of production of the original data,

history – a string providing an audit trail for modifications to the original data,

references – a string containing information on web based references referring to the instrumentation and documentation,

comment – a string containing miscellaneous information about the data or the methods used to produce them .